S P E C I F I C A T I O N S

for a

2 manual and pedal pipe organ

for

Immanuel Lutheran Church

Wadena, Minnesota

MANUAL COMPASS:		CC to C4	61 notes

PEDAL COMPASS:		CCC to G	32 notes

CONSOLE:			2 manual and pedal console built to AGO specifications with stop tabs

above the upper manual

DISPLAY FRONT:		100 square feet of casework, twin facades on each side of balcony.  				Silver lacquered facade pipes.  See rendering for details.

PREP OF ORGAN SPACE:	By purchaser to specifications set forth by Moe Pipe Organ Company 				including, but not limited to, relocation of sound system, installing AC

electrical outlets.

PIPEWORK:			9 ranks from existing organ, 7 ranks from donated organ, 6 ranks of

vintage pipework, 2 ranks of new custom pipework.

Total of 24 ranks of pipework plus chimes

Great Organ - 32

16' Violoncello [D]

8' First Open Diapason (1-40 facade)				New		61 pipes

8' Second Open Diapason						New		61 pipes

8' Gedeckt 					Donated		Maple Grove	61 pipes

8' Salicional (sw)

8' Voix Celeste TC (sw)

8' Erzahler (1-12 from Gedeckt)		Donated 		Vintage	49 pipes

8' Erzahler Celeste TC			Donated		Vintage	49 pipes

4' Octave					Donated		Maple Grove	61 pipes

4' Concert Flute				Donated		Vintage	61 pipes

4' Erzahler					Donated		Vintage	12 pipes

2 2/3' Twelfth					Donated		Maple Grove	61 pipes

2' Fifteenth					Donated		Maple Grove	61 pipes

1 3/5' Nineteenth				Donated		Vintage	37 pipes

III Mixture 1 1/3'				From present organ			183 pipes

V Grand Cornet (registration aid)

8' Trumpet [D]

8' Krummhorn [D]

Chimes

Great to Great 16'

Great Unison Off

Great to Great 4'

Swell to Great 16'

Swell to Great 8'

Swell to Great 4'

Resonance to Great

Autopedal

Zimbelstern

Tremulant

MIDI A

MIDI B

MIDI Audio to Chancel

Swell Organ - 28

16' Lieblich Gedeckt

8' Geigen Principal	(1-12 flute)		Donated		Maple Grove	49 pipes

8' Stopped Flute				From present organ			61 pipes

8' Salicional					From present organ			61 pipes

8' Voix Celeste TC				Donated		Maple Grove	49 pipes

4' Geigen Octave				Donated		Vintage	12 pipes

4' Harmonic Flute				Donated		Maple Grove	61 pipes

4' Violina					From present organ			12 pipes

2 2/3' Nazard					Donated		Maple Grove

2' Chimney Flute				From present organ			61 pipes

1 3/5' Tierce TC  				Donated		Vintage	37 pipes

1 1/3' Larigot					From present organ			61 pipes

1' Fife

V Cornet (registration aid, draws 8', 4', 2 2/3', 2', & 1 3/5')

IV Plein Jeu [D]

16' Bassoon [D]

8' Trumpet					Donated		Maple Grove	61 pipes

8' Oboe [D]

8' Vox Humana [D]

4' Clarion					Donated		Maple Grove	12 pipes

Chimes

Swell to Swell 16'

Swell Unison Off

Swell to Swell 4'

Resonance to Swell

Tremulant

MIDI A

MIDI B

Resonance Organ - 10

(Floating)

8' Solo Diapasons IV 			(plays gt diap, pd diap, Oct @8', sw diap)

8' Solo Flutes V 			(plays gt ged, c. fl, sw ged, h.fl, r. fl all at 8' pitch)

String Chorus VI 			(plays Salicional/Celeste at 16',8',4')

Full Celestes X 			(plays Salicional/Celeste at 16,8,4 and erzahler/celeste at 8,4)

16' Trompette de Reredos [D]

16' Fanfare Trumpet TC

8' Trompette de Reredos [D]

8' Fanfare Trumpet				Donated		Vintage		49 pipes

4' Trompette de Reredos [D]

4' Fanfare Trumpet				Donated		Vintage		12 pipes

Pedal Organ - 27

32' Contra Bourdon [D]

16' Open Diapason [D]

16' Bourdon					Donated		Maple Grove	12 pipes

16' Violoncello (gt)

16' Lieblich Gedeckt				From present organ			32 pipes

8' Octave (gt)	(1-40 facade)

8' Open Bass (gt)

8' Bass Flute (sw)

8' Gedeckt (gt)

4' Choral Bass	(gt)

4' Flute (sw)

II Grave Mixture

32' Contra Trombone [D]

16' Trombone [D]

16' Bassoon (sw)

8' Tromba [D]

8' Trumpet (sw)

8' Oboe (sw)

4' Clarion (sw)

4' Krummhorn (gt)

Great to Pedal 8'

Great to Pedal 4'

Swell to Pedal 8'

Swell to Pedal 4'

Resonance to Pedal

MIDI A

MIDI B

Combination Action (presets)

8 Pistons affecting the Swell stops

Swell Cancel

8 Pistons affecting the Great stops

Great Cancel

4  Toe studs affecting the Pedal stops

Pedal Cancel

8 General pistons affecting the entire organ (5-8 duplicated by toe studs)

General Cancel

Set Piston

32 memory levels

Piston Sequencer with Next/Previous Pistons and Toe studs

Accessories and Details

Balanced Swell Expression Pedal

Crescendo Pedal

Crescendo sequence is programmable

Crescendo indicator bar graph

Wind indicator light

Music Rack Light

Pedal Light

Adjustable bench

Great to Pedal Reversible Thumb and Toe

Full organ Reversible Thumb and Toe

Full-organ setting is programmable

Map piston for setting the effective range of the memory pistons

New solid-state rectifiers

Solid-state relay system with control panel

MIDI interfaces and connectors

MIDI Sustain toe stud

Includes a Syndyne Pro-Filer MIDI Sequencer

Includes Rodgers MX200 MIDI sound module

12 position Transposer

Pedal on divisonal piston controls for each division

Divisional cancels also controlled by bars above each section of stops

Up to 150 feet of cable between the console and the organ chambers

Includes steel I-beams and wooden beams to support the new organ independent of the

structure of the cantilevered portion of the balcony.

Casework includes a new balcony railing connecting the two portions of the organ case.

Balcony railing to include tempered glass panels for improved visibility.

Digital voices (marked with [D] in the stoplist) include necessary

amplifiers and speakers to be installed out of sight within the organ casework.

A pair of speakers will be installed out of sight and out of the way behind the reredos at

the front of the church to provide an outlet for the sounds produced by the MIDI

sound module when the organ is being used as part of a group of musicians

playing from the front of the church.